Indra-BMS

Edge-Native Differential Capacity Analysis (dQ/dV) for

Physics-Informed Battery Diagnostics on RISC-V

Transparent, Software-Defined Battery Intelligence for India's EV Ecosystem

| Team | Fumblers |
| --- | --- |
| Platform | VSDSquadron ULTRA (RISC-V) |
| Application Domain | Electric Vehicle Battery Management |
| Members | Arjun Bora and Arin Bhuyan |

# Executive Summary

Indra-BMS represents a paradigm shift in battery management systems for India's electric vehicle ecosystem. By implementing sophisticated electrochemical diagnostic algorithms directly on indigenous RISC-V processors, we transform battery health monitoring from a resource-intensive, opaque cloud service into a transparent, bandwidth-efficient edge intelligence platform.

Key innovations include:

- 99% bandwidth reduction: Transmitting 50-100 bytes of diagnostic features instead of 5 MB of raw telemetry per charge cycle

- Mechanistic transparency: Identifying specific degradation modes (Lithium Plating, Loss of Lithium Inventory, Loss of Active Material) rather than black-box predictions

- Indigenous silicon utilization: Optimized for RISC-V vector extensions and FPU, demonstrating Make-in-India capability

- 10-15% capacity extension: Physics-informed diagnostics enable targeted interventions to prolong battery life

# 1. Problem Statement

Current electric vehicle battery management systems in India rely on imported controllers that operate as opaque 'black boxes' with two critical deficiencies:

## 1.1 Bandwidth Inefficiency

Existing systems continuously upload raw sensor streams (voltage, current, temperature) to cloud platforms, consuming 2–5 MB per vehicle per hour. This approach creates:

- Prohibitive data costs for large-scale fleet deployment

- Network infrastructure dependencies that limit rural/remote operation

- Privacy concerns with continuous telemetry streaming

## 1.2 Diagnostic Opacity

Generic machine learning models predict outcomes ("battery will fail soon") without mechanistic explanations. These systems cannot differentiate whether degradation stems from:

- Lithium Plating: Metallic lithium deposits on anode surface during fast charging

- Loss of Lithium Inventory (LLI): Lithium trapped in solid-electrolyte interface

- Loss of Active Material (LAM): Electrode structural degradation

This opacity forces fleet operators to apply conservative limits (capping at 80% State of Charge), effectively wasting 15–20% of usable battery capacity and reducing economic viability.

## 1.3 Proposed Solution

By implementing electrochemical diagnostic algorithms directly on indigenous RISC-V processors, Indra-BMS creates a transparent, bandwidth-efficient, and scientifically grounded battery management system that identifies not just when but why batteries degrade.

# 2. System Architecture

Indra-BMS employs a software-defined architecture that shifts diagnostic intelligence from cloud to edge. Rather than streaming gigabytes of raw telemetry, the VSDSquadron ULTRA performs real-time Incremental Capacity Analysis (ICA) using numerical differentiation to compute the dQ/dV curve during charge cycles.

## 2.1 Physical Layer

| Component | Specification |
| --- | --- |
| Voltage Measurement | High-Precision ADC (ADS1115 or oversampled internal ADC for 16-bit resolution) |
| Current Measurement | INA219 Current Sensor |
| Temperature Measurement | DS18B20 Digital Temperature Sensor |

## 2.2 Compute Layer: VSDSquadron ULTRA (RISC-V)

The computational pipeline executes five critical stages:

### Stage 1: Circular Buffer

Stores the last 60 seconds of (V, I, T) samples during charging at 10 Hz sampling rate, enabling real-time window-based analysis.

### Stage 2: Savitzky–Golay Filter

Smooths voltage curve while preserving peak features using:

- Polynomial order: 3

- Window size: 11 samples

- RISC-V FPU optimization for matrix operations

### Stage 3: Numerical Differentiator

Computes Incremental Capacity using central difference method:

IC = dQ/dV ≈ (I · Δt) / ΔV

where I is measured current, Δt is sampling interval, and ΔV is voltage change between samples during constant-current charging.

### Stage 4: Peak Detection Algorithm

Identifies local maxima in the IC curve using derivative sign change, extracting:

- Peak voltage positions (Vpeak)

- Peak heights (ICmax)

- Peak widths

### Stage 5: Health Vector Generator

Maps peak shift from baseline to State of Health (SoH) percentage and classifies degradation mode (LLI vs LAM) based on multi-peak pattern analysis.

## 2.3 Output Layer

The system generates a compact JSON packet (approximately 50 bytes) transmitted via UART/WiFi:

{   "VID": "EV-01",   "SOH": 87.3,   "Mode": "LLI",   "P1_V": 3.42,   "P2_V": 3.68,   "RUL_cycles": 450 }

## 2.4 Cloud Layer: Fleet Fingerprint Registry

The optional cloud component:

- Aggregates health vectors from multiple vehicles

- Visualizes dQ/dV 'chemical fingerprints' as trend graphs

- Generates maintenance alerts when peak shifts exceed thresholds

# 3. Firmware & Algorithm Implementation

## 3.1 Signal Conditioning

We implement a Savitzky–Golay filter optimized for RISC-V vector extensions. Unlike simple moving averages that distort peaks, this polynomial filter smooths noise while preserving the sharp features in the IC curve that indicate electrochemical transitions.

Technical Rationale:

Differentiation amplifies noise; raw dV data can exhibit ±50 mV jitter from ADC quantization. The Savitzky–Golay filter acts as a differentiating smoothing operator, reducing noise by approximately 40 dB while maintaining phase transition peaks critical for diagnostic accuracy.

## 3.2 Core Diagnostic Logic

The system calculates Incremental Capacity as:

IC = dQ/dV ≈ (I · Δt) / ΔV

Electrochemical Foundation:

During lithium intercalation into graphite anodes, the electrochemical potential exhibits distinct plateaus corresponding to different staging phases (LiC₁₂, LiC₆). These appear as peaks in the IC curve. As the battery ages:

- Lithium loss shifts peaks to higher voltages

- Active material loss reduces peak heights

Since electrochemical peaks shift with temperature (Nernst Equation), Indra-BMS implements a Thermal Normalization Layer.

- We use an Arrhenius-based correction factor to normalize all dQ/dV curves to a standard 25℃ baseline before analysis.

- This ensures that a hot summer day doesn't trigger a false 'Aging Alert', a common failure point in standard voltage-threshold BMS.

## 3.3 RISC-V Optimization

| Optimization | Implementation |
| --- | --- |
| FPU Utilization | Floating-point matrix multiplication for Savitzky–Golay coefficients executes in hardware rather than soft-float, achieving 3–5× speedup |
| Vector Extensions | SIMD operations for element-wise array processing in filter convolution (if available) |
| Memory Management | Fixed-point arithmetic for non-critical paths to conserve RAM; floating-point only for differentiation kernel |

We utilize the RISC-V 'M' Extension (Integer Multiplication/Division) to accelerate the polynomial convolution steps of the Savitzky-Golay filter.

- Single-Cycle MAC: We map the filter's matrix operations to single-cycle Multiply-Accumulate instructions, reducing the 'Compute Cost per Sample' from 45 cycles (soft-float) to 4 cycles (hardware-accelerated)

## 3.4 Degradation Mode Classification

We employ a lightweight decision tree (trained offline on Oxford Battery Degradation Dataset) that maps peak shift patterns to degradation mechanisms:

| Degradation Mode | Signature |
| --- | --- |
| LLI (Loss of Lithium Inventory) | All peaks shift right (higher voltage) proportionally |
| LAM_PE (Loss of Active Material - Positive Electrode) | Peak heights decrease, positions remain stable |
| LAM_NE (Loss of Active Material - Negative Electrode) | Peaks shift left, heights decrease |

Output Example:

"73% capacity fade attributed to LLI + 27% LAM_PE"

Instead of opaque: "battery at 85% health."

# 4. Validation & Testing Strategy

## 4.1 Phase 1: Simulation & Algorithm Validation

Dataset: Oxford Battery Degradation Dataset (cells cycled to 80% capacity over 100+ cycles)

Benchmark: RISC-V firmware-generated IC curves compared against lab-grade potentiostat reference curves

Success Metrics:

- Peak position error < 20 mV

- SoH estimation error < 3%

## 4.2 Phase 2: Hardware-in-the-Loop

Test Cell: Single 18650 lithium-ion cell (LG INR18650 MJ1 or Samsung 30Q)

Procedure:

- Perform controlled charge cycle at 0.5C constant current

- VSDSquadron samples V/I/T at 10 Hz

- Compute IC curve in real-time

- Compare edge-computed features vs. offline Python analysis

Validation Target: Demonstrate <100 ms latency for complete IC computation on RISC-V

## 4.3 Phase 3: Multi-Cycle Aging Simulation

Methodology:

- Accelerate aging by cycling cell at elevated temperature (45°C) for 20–30 cycles

- Track peak shift over cycles to demonstrate RUL prediction capability

- Document correlation between peak voltage shift and capacity fade measured by standard discharge test

# 5. Expected Outcomes & Impact

## 5.1 Technical Deliverables

- Open-source firmware library: Complete C/C++ implementation of ICA algorithm for VSDSquadron ULTRA, published on GitHub under MIT license—the first dQ/dV library optimized specifically for Indian RISC-V processors

- Hardware reference design: Schematics and Bill of Materials (BOM) for precision voltage sensing module compatible with indigenous boards

- Cloud dashboard: React-based Fleet Fingerprint Registry showing real-time chemical health visualization across multiple vehicles

## 5.2 Innovation Impact

| Innovation Area | Value Proposition |
| --- | --- |
| Transparency | A 'white-box' diagnostic tool that explains why batteries age (e.g., "Anode graphite degradation detected"), enabling targeted interventions like modified charging profiles to extend life by 10–15% |
| Scalability | Bandwidth-efficient protocol transmitting 50 bytes per charge cycle vs. 5 MB of raw logs—enabling economically viable monitoring of million-vehicle fleets |
| Indigenous Capability | Demonstrates that sophisticated battery physics can run on Make-in-India silicon, reducing dependency on foreign BMS controllers |
| Talent Development | Creates reference implementation for future engineers working with RISC-V in automotive applications |

## 5.3 Commercial Pathway

This platform directly addresses predictive battery health analytics while laying the technical foundation for fleet-level dashboards. The physics-informed approach provides differentiable intellectual property that can evolve into:

- SaaS model for fleet operators: ₹500–1000 per vehicle per year for advanced diagnostics

- Firmware licensing to Indian EV manufacturers

- Second-life battery assessment service: Export degradation fingerprints for energy storage repurposing

# 6. Key System Advantages

| Feature | Traditional BMS | Indra-BMS |
| --- | --- | --- |
| Data Transmission | 2–5 MB per vehicle per hour | 50–100 bytes per charge cycle |
| Diagnostic Capability | Black-box prediction ("battery will fail soon") | Mechanistic explanation ("73% LLI, 27% LAM_PE") |
| Operating Mode | Cloud-dependent | Offline-capable with optional cloud |
| Processing Location | Cloud servers | Edge (RISC-V processor) |
| Hardware Dependency | Imported controllers | Indigenous RISC-V silicon |
| Capacity Utilization | Conservative limits (80% SoC cap) | Optimized based on actual degradation mode |

# Conclusion

Indra-BMS represents more than a technical innovation—it embodies a strategic vision for India's electric vehicle ecosystem. By implementing physics-informed diagnostics on indigenous RISC-V processors, we demonstrate that sophisticated electrochemical analysis need not be confined to cloud infrastructure or proprietary foreign controllers.

This proposal addresses critical gaps in battery management: the bandwidth inefficiency of continuous telemetry, the opacity of black-box machine learning, and the dependency on imported technology. Through Incremental Capacity Analysis executed at the edge, we transform battery health monitoring into a transparent, scalable, and scientifically grounded service that benefits fleet operators, EV manufacturers, and the broader Make-in-India initiative.

The path forward combines rigorous validation against established datasets, hardware-in-the-loop testing with commercial cells, and multi-cycle aging simulations. Upon completion, Indra-BMS will deliver not just a functional prototype but a comprehensive reference implementation—complete with open-source firmware, hardware schematics, and cloud visualization tools—that establishes a new standard for indigenous battery intelligence.